# Primary/secondary codes

is there any other signal like Galileo E1 having a code rate equals to the symbol/bit/secondary code rate ?

If by this you mean the primary ranging-code period is exactly equal to one navigation-symbol duration or one secondary/overlay-code chip duration, Galileo E1 is not unique.

The clearest other example is GPS L1C.

| Signal | Primary-code period | Data symbol / secondary-chip duration | Relationship |
| --- | --- | --- | --- |
| Galileo E1-B | 4 ms | 4 ms data symbol | 1 code = 1 symbol |
| Galileo E1-C | 4 ms | 4 ms secondary chip | 1 code = 1 secondary chip |
| GPS L1C-D | 10 ms | 10 ms data symbol | 1 code = 1 symbol |
| GPS L1C-P | 10 ms | 10 ms overlay chip | 1 code = 1 overlay chip |
| BeiDou B1C-data | 10 ms | 10 ms data symbol | 1 code = 1 symbol |
| BeiDou B1C-pilot | 10 ms | 10 ms secondary chip | 1 code = 1 secondary chip |
| Galileo E5a-Q | 1 ms | 1 ms secondary chip | 1 code = 1 secondary chip |
| GPS L5-Q | 1 ms | 1 ms NH20 chip | 1 code = 1 overlay chip |

### GPS L1C is particularly similar to Galileo E1

GPS L1C has a 10,230-chip primary code at 1.023 Mcps:

The L1C data symbols are at 100 symbols/s, hence:

and the L1C pilot overlay code is also clocked at 100 chips/s, giving:

Therefore:

The GPS documentation explicitly describes one data symbol as spreading exactly one L1C-D primary-code block, while one L1C-O overlay chip modulates exactly one L1C-P primary-code block.

### BeiDou B1C does essentially the same thing

BDS B1C also uses a 10-ms primary code (10,230 chips at 1.023 Mcps). Its pilot secondary code contains 1800 chips and lasts 18 s, so:

Thus again:

The B1C data channel also operates at 100 symbols/s, so one data symbol corresponds to one 10-ms primary-code epoch.

### In fact, this is the normal tiered-code construction

For modern pilot GNSS signals, having one secondary-code chip per primary-code epoch is extremely common. Galileo E5 illustrates this nicely:

- E5a-Q: 1-ms primary × 100-chip secondary → 100 ms

- E5a-I: 1-ms primary × 20-chip secondary → 20 ms

- E5b-Q: 1-ms primary × 100-chip secondary → 100 ms

- E5b-I: 1-ms primary × 4-chip secondary → 4 ms

The secondary-code chip rate is therefore 1 kchip/s, exactly the primary-code repetition rate.

So if you're considering this from the acquisition/power-spreading problem we discussed earlier, the interesting distinction isn't really Galileo E1 versus other modern signals. GPS L1C and BeiDou B1C have exactly the same structural property, while GPS L5 and Galileo E5 have it on a much shorter, 1-ms timescale.

# E5a F/NAV

When E1 and E5a are both enable gnss-sdr requires F/NAV epehemeris to produce a fix.

Make sure the aiding you provide gal_fnav_ephemeris.xml instead of gal_inav_ephemeris.xml as initial ading.